## Appendices

## Appendix A:	Reimbursements

This form is to be submitted with your design report.

This year, any monies due to competitors will be paid to a representative of your university. Examples of valid representatives are your faculty advisor or your departmental accounting administrative assistant. Unacceptable representatives include students, parents, friends, etc.

Please provide complete contact information for this representative.

| School |  |
| --- | --- |
| Name of Representative |  |
| Position at Institution |  |
| Complete Mailing Address |  |
| Phone |  |
| Email Address |  |

## Appendix B:	Material Specifications

- Sand:

  - Clean sand with grain size distribution as specified in Table 1 and Figure 3

  - Grain shape will be rounded to sub-rounded

- Sandbox Material:

  - Walls and Base:  23/32 or 3/4ʺ plywood, any grade

  - Tie Rod:  ¼˝ threaded steel rod with washers and nuts as needed

  - Fasteners:  any suitable wood fasteners

- Facing Material:

  - Poster Board, 22" x 28", White

  - Grammage: 194 g/m2, 0.125 g/in2

  - Office Depot® Item # 858277 (Pack Of 10)

- Reinforcing Material:

  - 60 lb Kraft Paper

  - Grammage: 97.7 g/m2, 0.063 g/in2

  - Office Depot® Postal Wrap Item # 444835 (2' x 50' roll)

- Adhesive Material:

  - Heavy duty, clear, 2˝ wide, polypropylene package tape

  - Scotch® 142-B Super Strength Mailing Tape, clear

  - Office Depot® Item #650457, 2" x 22.2 Yd with dispenser

- Rectangular Vertical Surcharge Bucket:

  - Sterilite TouchTop Waste Basket, 7.5 gal

  - Walmart SKU #073149104380

## Appendix C:	Design Report Judging Rubric

| Geo-Institute of the ASCE: GeoWall Design Paper – Scoring Form |  |  |  |
| --- | --- | --- | --- |
| Reviewer Guidelines: 1) Place weight on the team ability for engineering reasoning not technical knowledge; 2) Place weight on team communication skills on procedures, findings and observations; 3) Score in 0.5-point increments; and 4) Team to be awarded higher score if design parameters were verified |  |  |  |
| Criterion | Max | Actual | Notes |
| 1) Formatting, Mechanics, Grammar & Safety |  |  |  |
| a. Paper length, margins & font are acceptable | 1 |  | Paper complies with specifications |
| b. Layout, or structure, of paper is logical | 1 |  | Paper organization is clear and supports the message. |
| c. Grammar and punctuation are correct | 2 |  | Error free paper with writing that clearly presents design. |
| d. Figures & tables are clear, properly numbered, captioned and referenced in the text | 2 |  | Good choice of tables vs. figures, clear presentation of data. |
| e. References are reasonably formatted and complete | 1 |  | Quantity appropriate with correct citations and references |
| f. Appendix A and safety appendix (Appendix E) complete with reasonable controls | 2 |  | Clearly identifies key safety concerns and provides viable plans to keep team safe during competition. |
| 2) Experimental Methods, Analyses and Design |  |  |  |
| a. Methods to obtain soil properties | 3 |  | Experimental methods are reasonable and clearly described |
| b. Methods to determine reinforcement properties | 3 |  | Experimental methods are reasonable and clearly described |
| c. Methods to determine backfill-reinforcement interaction | 3 |  | Experimental methods are reasonable and clearly described |
| d. Engineering properties are reasonable | 3 |  | Backfill unit weight, friction angle, interface friction angle, reinforcement strength are compared to typical values |
| e. Earth-pressure calculations (backfill only) | 3 |  | Calculations are correct and presented in a logical, readily followed format |
| f. Vertical surcharge load included in the design | 3 |  | Considers lateral loads on wall and effect on reinforcement pullout |
| g. Method used to account for eccentrically applied loads | 3 |  | Model and assumptions are reasonable |
| h. Method used to account for 3-D wall geometry | 3 |  | Method and assumptions are reasonable |
| i. Determination of reinforcement length | 3 |  | Model accounts for 3-D geometry and is reasonable and appropriate |
| j. Determination of reinforcement spacing | 3 |  | Method and assumptions are reasonable |
| k. Evaluation of connection strength | 3 |  | Method and assumptions are reasonable |
| 3) Engineering Reasoning and Communication |  |  |  |
| The report is, on the whole, clear, precise, and well-reasoned. Engineering terms and distinctions are used effectively and in keeping with established professional usage. The report demonstrates a clear and precise analysis of the MSE wall design problem, very little or no irrelevant information is presented, key assumptions are identified, and key concepts are clarified. The authors have shown, through their report, excellent engineering reasoning and problem-solving skills. | 10 |  | Scores may range from 0 to 10. It is the opinion of the reviewer as to how the overall report measures up to the criteria listed under item 3, "engineering reasoning and communication". |
| Total | 50 |  |  |

## Appendix D:	Judges’ Scoring Checklist for GeoWall Competition

## D1: Captains’ meeting—Box check

| Team School: |  | Deductions |  |
| --- | --- | --- | --- |
| Item | Instruction | Minor | Major |
| Plywood | - 23/32 or ¾˝ thickness - Inside surfaces planar and natural |  |  |
| Box dimensions | - Within tolerance - Sand fill height marked |  |  |
| Facing panels | - Flush to box base - Removable fasteners - Base extends to outside of vertical facing panels |  |  |
| Tie rod | - ¼˝ - inch diameter - Located within tolerances |  |  |
| Measuring frame attachment | - Frame fits properly on box |  |  |
| Tools | - Only authorized tools used |  |  |
| Other minor, explain: |  |  |  |
| Other major, explain: |  |  |  |
| Disqualification, explain: |  |  |  |
|  | Total deductions |  |  |

Notes:

## D2: Reinforcement fabrication

| Item | Instruction |  | Time |  |
| --- | --- | --- | --- | --- |
|  |  |  | Total | > 15:00 (min:sec ) |
| Time | Give start command. Time ends when all elements cut to size and shape |  |  |  |
|  |  |  | Mass (g) |  |
|  |  |  | Design | Actual |
| Mass | Weigh reinforcement to nearest 0.01 g |  |  |  |
| Compute official adjusted Mass, M, using Equation 2 |  |  | M = |  |
|  |  |  | Deductions |  |
| Deductions |  |  | Minor | Major |
| Tools | Only authorized tools used |  |  |  |
| Safety | No mishaps |  |  |  |
| Other, explain |  |  |  |  |
|  | Total deductions |  |  |  |

Notes:

## D3: Wall Assembly

| Team School: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Instruction |  | Time |  |
|  |  |  | Total | > 15:00 (min:sec ) |
| Time | Give start command. Time ends when wall is assembled and trial fitted to box (NO SAND PLACED DURING THIS PHASE) |  |  |  |
|  |  | Deductions |  |  |
|  |  | Minor |  | Major |
| Facing construction | - Single lap joint 1” wide - Trimmed ~3/16” below top of wall |  |  |  |
| Reinforcement attachment | - Each tape piece ≤ (2˝ 2˝) - On vertical front plane only - Not overlapping - Touches both wall and reinforcement |  |  |  |
| Tools | - Only authorized tools used |  |  |  |
| Safety | - No mishaps |  |  |  |
|  | Total deductions |  |  |  |

Notes:

## D4: Construction

| Item | Instruction | Time |  |
| --- | --- | --- | --- |
|  |  | Total | > 20:00 (min:sec ) |
| Time | Give start command. Time ends when soil filled to line and empty loading platforms are in place |  |  |
|  |  | Deductions |  |
|  |  | Minor | Major |
| Backfill | - Level - Filled to fill line |  |  |
| Tools | - Only authorized tools used |  |  |
| Safety | - No mishaps |  |  |
|  | Total deductions |  |  |

Notes:

## D5: Loading

| Team School: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Item | Instruction |  |  |  |  |
| Stage 1: Backfill only | - Place clean poster board on floor in front and sides of box - At judge’s direction students remove panels from box. Electric drills/screwdriver may be used to remove fasteners. - Once panels are completely removed start 1 min wait period - Attach measuring frame - At end of 1 min, make following checks |  |  |  |  |
|  | - Swipe front wall front and sides with straight edge to check wall deflection |  | - Pass |  | - Fail D = 5 |
|  | - Less than 30 cm3 sand leaked from box onto floor |  | - Pass |  | - Fail Major Ded |
|  | - Catastrophic failure |  | - Pass |  | - Fail DQ |
| Stage 2: Vertical Surcharge | - Bucket pre-weighed with 50 lbs of sand should be ready. - At judge’s direction students add 50 lbs of sand to surcharge platform. Students have one minute to complete loading. - Once load is placed start 1 min wait period - At end of 1 min, make following checks |  |  |  |  |
|  | - Loading complete within 1 minute |  | - Yes |  | - No Minor Ded |
|  | - Swipe front wall face with straight edge to check wall deflection |  | - Pass |  | - Fail D = 3 |
|  | - Less than 30 cm3 sand leaked from box onto floor |  | - Pass |  | - Fail Major Ded |
|  | - Catastrophic failure |  | - Pass |  | - Fail DQ |

## D6: Scoring

Adjusted mass, M, computed by

Score=R+1520-M-10Nmin-40Nmaj-2T-20D

| Team School: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Item |  | Score | Weight |  | Extended |
| Report score out of 50, R |  |  | 1 |  |  |
| Reinforcement mass score, enter as (20 – M) |  |  | 15 |  |  |
| Total # of minor deductions, Nmin |  |  | -10 |  |  |
| Total # of major deductions, Nmaj |  |  | -40 |  |  |
| Total time over limit rounded up to nearest whole minute, T |  |  | -2 |  |  |
| Deflection rating, D 5 = Deflection exceeded at Stage 1 3 = Deflection exceeded at Stage 2 0 = Deflection never exceeded |  |  | -20 |  |  |
| Catastrophic failure any stage disqualifies the team |  | DQ | Stage # |  |  |
|  |  |  | Final Score |  |  |

Notes:

## Appendix E: Safety Appendix

This section is intended for each team to consider the competition steps and manage safety risk. Use rows as necessary.

| Title | Work Task | Hazards | Controls |
| --- | --- | --- | --- |

Notes:

- Safety mishaps that result in bleeding will be classified as “major.”

## Appendix F:	Bio-form to be completed by each team captain and submitted to the head judge at the pre-competition meeting

Geo-Institute of ASCE GeoCongress 2020

GeoWall Competition Bios

Team School:

Team Mascot:

No. of Years Competing at Nationals:

Team Advisor:

Team Captain:

Current Year in School (junior, senior, MS, or PhD):

Hometown (City and State or Country)

Other School Activities:

Interests/Hobbies:

Future Plans, e.g., graduate school, consulting, government, other?

Geographical preferences?

## Appendix G:	Bio-form to be completed by each team member and submitted to the head judge at the pre-competition meeting

Geo-Institute of ASCE GeoCongress 2020

GeoWall Competition Bios

Team School:

Team Mascot:

No. of Years Competing at Nationals:

Team Advisor:

Team Member:

Current Year in School (junior, senior, MS, or PhD):

Hometown (City and State or Country)

Other School Activities:

Interests/Hobbies:

Future Plans, e.g., graduate school, consulting, government, other?

Geographical preferences?